Warehouse Sensor Node

Setup and Manual Testing Runbook

Fresh clone to full Arduino, bridge, Docker dashboard, and manual demo test

Purpose: Use this document to set up the project from GitHub, start the dashboard and bridge, flash the Arduino firmware, and perform the complete voice, colour, and movement manual test. It is written for a teammate or examiner who has not used the repo before.

| Item | Value |
| --- | --- |
| Repository | https://github.com/serhiiSotskyi/Iot |
| Primary local dashboard URL | http://localhost:3001/ |
| Hardware target | Arduino Nano 33 BLE / Nano 33 BLE Sense |
| Serial baud rate | 115200 |
| Main success path | setup_status -> voice_start -> colour_authenticated -> movement -> session_complete |

# Contents

1. Start from GitHub

2. Configure the local environment

3. Start the Docker dashboard and database

4. Flash the Arduino firmware

5. Start the Python serial bridge

6. Manual test procedure

7. Verification commands

8. Troubleshooting and recovery

9. Shutdown and reset checklist

# 1. Start from GitHub

Begin from a fresh clone of the repository. The steps below assume macOS first because the current manual demo uses Docker Desktop and Arduino serial devices named /dev/cu.usbmodem*. Linux and Windows port notes are included later.

## 1.1 Prerequisites

Git installed and available from the terminal.

Docker Desktop installed and running before starting the web stack.

Python 3.10 or newer. Python 3.11 is known to work with the bridge virtual environment.

Arduino IDE 2.x installed for flashing the sketch.

Arduino Nano 33 BLE / Nano 33 BLE Sense with a USB data cable. Charging-only cables will not work.

A green object or card for the colour authentication stage.

A microphone environment quiet enough to say the word Start clearly after reset.

## 1.2 Clone the repository

git clone https://github.com/serhiiSotskyi/Iot.git cd Iot

Current main note: This runbook is self-contained and does not rely on the old docs/ folder or scripts/manual-test.ps1. Use the files that remain in the current main branch: README.md, docker-compose.yml, .env.example, _update_server, verify.sh, bridge/, arduino/, web/, and tests/.

# 2. Configure the local environment

The Docker stack requires bridge and admin tokens. Dashboard login is also recommended for manual testing because it exercises the same authentication path used during the demo.

## 2.1 Create .env

cp .env.example .env

## 2.2 Generate secrets

Run the command below four times, or run it once for each secret you need to fill. Use different values for BRIDGE_API_TOKEN, ADMIN_API_TOKEN, and SESSION_SECRET.

openssl rand -hex 32

## 2.3 Recommended .env values for local manual testing

POSTGRES_DB=iot_demo POSTGRES_USER=iot_demo POSTGRES_PASSWORD=iot_demo_password WEB_PORT=3001 POSTGRES_HOST_PORT=5434 INGEST_RATE_LIMIT_PER_MIN=0 DASHBOARD_PASSWORD=choose-a-demo-password SESSION_SECRET=<paste openssl rand -hex 32 output> BRIDGE_API_TOKEN=<paste openssl rand -hex 32 output> ADMIN_API_TOKEN=<paste openssl rand -hex 32 output>

| Variable | What it controls |
| --- | --- |
| WEB_PORT | Host port for the dashboard. Use 3001 if 3000 is busy or if the browser is already open at localhost:3001. |
| POSTGRES_HOST_PORT | Host port for Postgres. Use 5434 if local Postgres already uses 5432. |
| DASHBOARD_PASSWORD | Password used on the dashboard login screen. |
| SESSION_SECRET | HMAC key for the dashboard session cookie. Must be at least 16 characters. |
| BRIDGE_API_TOKEN | Bearer token the Python bridge sends to /api/movement and /api/bridge/control. |
| ADMIN_API_TOKEN | Bearer token accepted by the Stop endpoint for non-browser/admin calls. |

# 3. Start the Docker dashboard and database

Make sure Docker Desktop is already running. Then build and start the production-like local stack from the repository root.

docker compose up -d --build docker compose ps

## 3.1 Expected Docker result

The command should show both containers running. With WEB_PORT=3001, the web port should look like 0.0.0.0:3001->3000/tcp.

NAME STATUS PORTS iot-demo-postgres Up ... (healthy) 127.0.0.1:5434->5432/tcp iot-demo-web Up ... (healthy) 0.0.0.0:3001->3000/tcp

## 3.2 Open the dashboard

Open http://localhost:3001/ in the browser. If DASHBOARD_PASSWORD is set, the app redirects to /login. Sign in with the value from .env.

## 3.3 Useful Docker commands

docker compose logs -f web docker compose logs -f postgres docker compose restart web docker compose down

Server readiness: Do not start the bridge until Docker Compose reports the web container as running or healthy. If the bridge starts while the web app is unavailable, it will queue events and may hit the current retry caveat described in Troubleshooting.

# 4. Flash the Arduino firmware

The board does not connect to Wi-Fi. It runs the three TinyML models locally and prints JSON over USB Serial. The Python bridge converts those JSON lines into HTTP requests.

## 4.1 Open the sketch

arduino/voice_colour_motion_demo/voice_colour_motion_demo.ino

## 4.2 Install required Arduino libraries

Arduino_APDS9960

Arduino_LSM9DS1

PDM

combined_inferencing from arduino/libraries/combined_inferencing/

In Arduino IDE, either set the sketchbook location to the repository arduino/ folder, or copy arduino/libraries/combined_inferencing into your normal Arduino libraries folder. Restart Arduino IDE after changing library locations.

mkdir -p "$HOME/Documents/Arduino/libraries" cp -R arduino/libraries/combined_inferencing "$HOME/Documents/Arduino/libraries/"

Important: Do not install the individual Edge Impulse export folders as separate Arduino libraries for the main sketch. The sketch is designed to compile against the combined_inferencing library, which exposes all three impulse handles without duplicate SDK symbols.

## 4.3 Board and upload settings

Select Arduino Nano 33 BLE or Nano 33 BLE Sense, depending on the connected board package name.

Select the serial port shown by Arduino IDE.

Upload the sketch.

After upload, close Arduino Serial Monitor before running the Python bridge. Only one process can own the serial port.

## 4.4 Reset behavior

| Action | Meaning for the demo |
| --- | --- |
| Single press reset | Restarts the currently flashed sketch. Use this before a clean manual test. |
| Double press reset | Enters bootloader mode. Use only if the board is not accepting uploads. |
| Restart Python bridge only | Does not reset the Arduino state machine. The board may remain in WAITING_FOR_COLOUR or TRACKING_MOVEMENT. |

# 5. Start the Python serial bridge

The bridge reads newline-delimited JSON from the Arduino at 115200 baud and posts each valid event to the dashboard API. Start the web stack first, then start the bridge.

## 5.1 Create the Python environment

python3 -m venv .venv source .venv/bin/activate pip install -r bridge/requirements.txt

## 5.2 Find the serial port

ls /dev/cu.usbmodem*

Typical macOS output is /dev/cu.usbmodem1201. If no usbmodem device appears, unplug and replug the board, try another cable, or double-press reset to enter bootloader mode.

## 5.3 Export the bridge token

export BRIDGE_API_TOKEN="$(grep '^BRIDGE_API_TOKEN=' .env ¦ cut -d= -f2-)"

## 5.4 Start the bridge

python bridge/serial_to_http.py \ --port /dev/cu.usbmodem1201 \ --endpoint http://localhost:3001/api/movement \ --api-token "$BRIDGE_API_TOKEN" \ --verbose

Clean run sequence: Start the bridge, then single-press reset the Arduino. The first important event should be setup_status. If the first event is voice_debug, colour_debug, or movement, the board is probably still in a later state from a previous run.

## 5.5 Optional debug output

The bridge drops voice_debug and colour_debug by default. For manual tuning, forward every debug event with --debug-downsample 1.

python bridge/serial_to_http.py \ --port /dev/cu.usbmodem1201 \ --endpoint http://localhost:3001/api/movement \ --api-token "$BRIDGE_API_TOKEN" \ --debug-downsample 1 \ --verbose

# 6. Manual test procedure

This is the full physical walkthrough. Keep the dashboard visible in the browser and the bridge terminal visible beside it.

| Step | Action | Expected result |
| --- | --- | --- |
| 1 | Start Docker and confirm the dashboard is open at http://localhost:3001/. | Login screen or dashboard loads successfully. |
| 2 | Start the Python bridge with the correct serial port and token. | Terminal prints polling control endpoint and opening serial port. |
| 3 | Single-press reset on the Arduino. | Bridge posts setup_status -> 200. Dashboard shows Sensor node booted. |
| 4 | Wait for the 4-second arm delay, then say Start clearly. | Bridge posts voice_start -> 200. Dashboard shows "Start" voice authentication detected. |
| 5 | Hold the green object/card close to the APDS-9960 colour sensor. | Bridge posts colour_authenticated -> 200. Dashboard shows Green color detected. |
| 6 | Move the board up, down, left, or right. | Bridge posts movement -> 200. Dashboard shows Detecting movement and a motion confidence value. |
| 7 | Click End pick session on the dashboard. | Session is saved as completed if voice and colour passed. Bridge exits after stop request. |
| 8 | Open the recent pick session in the dashboard list. | Replay timeline shows voice, colour, movement, and session close events. |

## 6.1 Expected bridge lines

[bridge] POST setup_status -> 200 [bridge] POST voice_start -> 200 [bridge] POST colour_authenticated -> 200 [bridge] POST movement -> 200 [bridge] stop requested by server; closing serial connection

## 6.2 Expected dashboard labels

Sensor node booted

"Start" voice authentication detected

Green color detected.

Detecting movement.

Pick session closed

# 7. Verification commands

Run these after the manual stack is up. They do not replace the physical Arduino test, but they confirm that the web auth boundary and bridge parsing logic are working.

## 7.1 API and auth verification

./verify.sh http://localhost:3001

Expected result: all checks pass. The script needs DASHBOARD_PASSWORD, SESSION_SECRET, BRIDGE_API_TOKEN, and ADMIN_API_TOKEN in .env.

## 7.2 Bridge unit tests

python -m unittest tests.bridge.test_serial_to_http -v

Expected result: all tests pass. These tests cover JSON-line parsing, control endpoint inference, and colour-auth endpoint selection.

## 7.3 Manual API checks

# Latest event requires dashboard cookie when login auth is enabled. curl -i http://localhost:3001/api/latest # Bridge control requires BRIDGE_API_TOKEN. curl -i http://localhost:3001/api/bridge/control \ -H "Authorization: Bearer $BRIDGE_API_TOKEN" # Manual ingest test. curl -i -X POST http://localhost:3001/api/movement \ -H "Authorization: Bearer $BRIDGE_API_TOKEN" \ -H "Content-Type: application/json" \ -d '{"event":"setup_status","voiceReady":true,"colourReady":true,"imuReady":true}' # Complete active session with ADMIN_API_TOKEN. curl -i -X POST http://localhost:3001/api/sessions/current/complete \ -H "Authorization: Bearer $ADMIN_API_TOKEN"

Cookie note: The browser dashboard uses a signed iot_session cookie after login. Plain curl requests to read-only dashboard APIs can return 401 unless you log in with curl and keep the cookie jar.

# 8. Troubleshooting and recovery

| Symptom | Likely cause | Recovery |
| --- | --- | --- |
| Connection refused for localhost:3001 | Docker web container is stopped, still restarting, or WEB_PORT is different. | Run docker compose ps. Wait until iot-demo-web is healthy/running. Use the port shown in PORTS. |
| 401 Unauthorized from bridge or API | Missing or wrong BRIDGE_API_TOKEN. | Re-export BRIDGE_API_TOKEN from .env and pass --api-token "$BRIDGE_API_TOKEN". |
| stop requested by server after colour_debug or voice_start | Server is paused waiting for a fresh setup_status after a previous Stop. | Start bridge, single-press reset the board, and wait for setup_status -> 200 before saying Start. |
| Device not configured | USB serial device disappeared, board reset into another mode, or cable disconnected. | Unplug/replug, rerun ls /dev/cu.usbmodem*, then restart bridge with the new port. |
| No serial data for 60s | Wrong port, board not printing JSON, sketch not uploaded, or Serial Monitor owns the port. | Close Serial Monitor, verify sketch upload, reset board, and restart bridge. |
| Dashboard shows old sessions | Postgres persists sessions across restarts. | Select the latest session or clear the Docker volume only if you intentionally want a blank database. |
| Bridge traceback: flush_retry_queue takes 4 positional arguments but 5 were given | Current bridge retry path has an argument mismatch when the API is down and queued events are flushed. | Workaround: start Docker first and wait healthy before starting bridge. Code fix is to remove args.endpoint from the flush_retry_queue call. |

## 8.1 Resetting the paused recording state

After End pick session, the server intentionally waits for a fresh setup_status. This avoids appending a new physical run onto an old session. The normal recovery is to reset the Arduino while the bridge is listening.

# Normal reset path # 1. Stop bridge with Ctrl+C if it is running. # 2. Start bridge again. # 3. Single-press reset on Arduino. # 4. Confirm POST setup_status -> 200.

## 8.2 Port notes for other operating systems

| Platform | Port command | Typical port |
| --- | --- | --- |
| macOS | ls /dev/cu.usbmodem* | /dev/cu.usbmodem1201 |
| Linux | ls /dev/ttyACM* /dev/ttyUSB* | /dev/ttyACM0 |
| Windows | Device Manager > Ports | COM6 or similar |

# 9. Shutdown and reset checklist

End the active pick session from the dashboard if the run passed voice and colour authentication.

Stop the bridge with Ctrl+C if it did not already exit after End pick session.

Stop Docker with docker compose down when finished.

For another clean demo, start Docker, start bridge, then single-press reset the Arduino before saying Start.

docker compose down

# Appendix A. Quick one-command server update

If the repository is already cloned on a server or laptop, _update_server pulls the latest main branch and rebuilds the Docker stack. It still requires .env to contain BRIDGE_API_TOKEN and ADMIN_API_TOKEN.

chmod +x ./_update_server ./_update_server